| FR |  |  |  | ge | According to STEP Manufacturing Suite, AP219 is similar for inspection purpose to AP238 for NC machining. AP219 defines inspection features from AP203 and GD&T; so it is similar to AP224. DMIS is for inspection similar to AP240. I++DME is similar to AP238. A new work item reduced the scope of AP219 to standardize the measurements results and provide the tools to analyze these results. AP219 –DIS does not fulfil this requirement. What happens for DML? | AP219 role must be defined at once and its interfaces (DMIS, I++ DME, DML and other) agreed by all the standardisation bodies and implementers. | Accepted. We agree with the comment. One function of AP219 is to translate the results of dimensional inspections, such as those driven by DMIS, into STEP concepts so that these results can be integrated into STEP manufacturing implementations. We have been working to harmonize AP219 with DMIS and DML and a number of DML elements and attributes are included in AP219. We need to rewrite the introduction to reflect this resulting harmonization and emphasize there that AP219 cannot be used for process planning. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| JP-01 (CS) | 10303-219 | Annex G | Page 1136 | ed | The title “Figure G.2” is not correct. | “Figure G.2”  “Figure G.5” | Accepted. |
| JP-02 (CS) | 10303-219 | Annex G | Page 1137 | ed | The title “Figure G.2” is not correct. | “Figure G.3”  “Figure G.6” | Accepted. |
| US-1 | 10303-219 | 4.2.75 | Attribute-specification | te | Consistency with other clauses | Move the attribute, specification, to Subclause 4.2.84, Dm_result_parameter, to be consistent with the ARM, Annex G | Accepted. |
| US-2 | 10303-219 | Annex G | Figure G.2 | te | Modify the choices for specification_select to include geometric tolerances | The choices should be Geometric_tolerance, Dimensional_tolerance, and Manufacturing_feature | Accepted. Revise the ARM so that the choices under Specification_select are Geometric_tolerance, Dimensional_tolerance, Manufacturing_feature, and Shape_element so that measured parameters can be compared with any tolerances, designed parameters of manufacuturing features, or parameters of additional features defined ad hoc. |
| US-3 | 10303-219 |  | 5.1.1.2.2 month | te | Consistency of dates information | Change “day_or_month_number” to “month_in_year_number” | Accepted. |
| US-4 | 10303-219 |  | 5.1.1.3.1 date | te | Consistency of dates information | Change “year_number” to “date” | Accepted. |
| US-5 | 10303-219 |  | 5.1.2.2 Dm_ analysis_dofs_dml | te | Question about the source of the AIM element , dm_analysis_dofs_dml | This element was defined in the development of AP219 and could not come from Part 41 | Accepted. |
| US-6 | 10303-219 |  | 5.1.2.3.1 lower_limit and 5.1.2.3.3 upper_limit | te | Distinguish the adjustment limits of a reference line or plane from a tolerance limit and a measurement uncertainty. | Rework the reference path for these attributes so that an adjustment limit is represented | Accepted. Refine both the ARM and AIM to make clear the differences between uncertainties in measured parameters, part tolerances, and adjustment limits |
| US-7 | 10303-219 |  | 5.1.3.1 Document_assignment | te | Clarify source of AIM element, “applied_document_reference” | The entities, “applied_document_reference,” and “document_reference” do not seem to come from AP219, yet AP219 is given as the source where “applied_document_ reference” is defined. This also conflicts with Subclause 5.1.3.2 where AP240 is given as the source of the same entity. | Rejected. Applied_document_reference is shown in AP219 AIM Diagram H.12 |
| US-8 | 10303-219 | 5.1 | Definition of [I] | ed | Improve clarity by avoiding an ambiguous usage of the word “following” | Change the definition to read “[i] attribute is an aggregation of which a single member is given in an adjacent row referenced by the -> or <- symbols.” | Rejected. Questions about instructions for Mapping Specifications are not under the scope of T24 and, if valid, should be referred to the Quality Committee. |
| US-9 | 10303-219 |  | 5.1.1.1.11 telex_number | ed | Consistency of dates information | Change “address.electronic_mail_address” to “telex_number” | Accepted. |
| US-10 | 10303-219 |  | 5.1.1.5.4 middle_names | ed | Editorial consistency | Change the AIM element to “person.middle_names” | Accepted. |
| US-11 | 10303-219 |  | 5.1.1.6.1 dirfection | ed | Typo | Change “dirfection” to “direction.” | Accepted. |
| US-12 | 10303-219 |  | 5.1.2.1 Dm_parameter_analysis_dml | ed | Entity out of order | Move Dm_parameter_analysis_dml into proper alphabetical order to be consistent with Section 4.1.2 | Accepted. |
| US-13 | 10303-219 |  | 5.1.2.3.1lower_limit and 5.1.2.3.3 upper_limit | ed | Typo | Dangling parenthesis on first line | Accepted. |
| US-14 | 10303-219 |  | 5.1.2.3.2 settings | ed | Typo | Change “settings” to “setting” for consistency with Subclause 4.2.114.2 setting | Accepted. |
| US-15 | 10303-219 |  | 5.1.2.3.2 settings | ed | Typo | Take out the line that just reads “type_qualifier.” It seems redundant. | Accepted. |
| US-16 | 10303-219 |  | 5.1.2.4.1 axis_method and 5.1.2.5.1 axis_method | ed | Typo | The words “representation.items[I] -> representation_item” are written twice, redundant, delete one. | Accepted. |
| US-17 | 10303-219 |  | 5.1.2.4.1 axis_method | ed | Typo | Change “feature analysis mode axis” to “feature analysis mode axis_method” | Accepted. |
| US-18 | 10303-219 |  | 5.1.2.4.1 axis_method | ed | typo | Change “descriptive” to “descriptive” in three places here and many other places. Do a global change for this. | Accepted |
| US-19 | 10303-219 |  | 5.1.2.4.6 and 5.1.2.4.7 | ed | Typo | Change “limits’)” to “limits of size’)” and “surface’)” to “surface analysis’)” | Accepted. |
| US-20 | 10303-219 |  | 5.1.2.4.8 Dm_feature_… | ed | Typo | Change “Dm_” to “dm_” in two places | Accepted. |
| US-21 | 10303-219 |  | 5.1.2.5.1 axis_method | ed | Typo | Change “axis default” to “axis method default” | Accepted. |
| US-22 | 10303-219 |  | 5.1.2.5.3 aelpr_analysis | ed | Typo | Change “defaul” to “default” | Accepted. |
| US-23 | 10303-219 |  | 5.1.2.5.6 limits_of_size | ed | Typo | Change “limits default” to “limits of size default” | Accepted. |
| US-24 | 10303-219 |  | 5.1.2.6.1 method | ed | Typo | Change “feature tolerance mode method” to “tolerance analysis mode method.” See also analogous changes in 5.1.2.6.2 option, 5.1.2.6.3 setting, 5.1.2.7, 5.1.2.7.1 method, 5.1.2.7.2 option, and 5.1.2.7.3 setting. | Accepted. |
| US-25 | 10303-219 | Annex G | Fig. G.4 | ed | Typo | Change the attribute “default_value” to “default_values” | Accepted. |
| US-26 | 10303-219 |  | 5.1.3.2.1 Part_dimensioning… | ed | Typo | Change “Part” to lower case. Also change “Part_version” to “part” for consistency with the object definitions and Figure G. 34 | Accepted. |
| US-27 | 10303-219 |  | 5.1.4.1dm_execution_ input | ed | Typo | Redundant with Subclause 5.1.4.2. Eliminate one heading and renumber | Accepted. |
| US-28 | 10303-219 |  | 5.1.4.2 Dm_execution_input | ed | Typos | Change “5.2.3.1.36” to “5.2.3.1.35”. Change “dm_execution_result” to “dm_execution_input” | Accepted. |