Draft

High-level Measurement Process Planning Glossary

Activities

A0 Perform High-level Measurement Process Planning (HiPP)

The overall activity to develop a high-level measurement process plan that defines measurement scope, optionally dimensional measure equipment list, optionally a sequence of high-level measurement operations, optionally a reaction plan, and optionally supporting data. A0 has supporting measurement databases and knowledge base for decision making, as well as software systems for process planning. A0 requires standards, company policies, manufacturing plans, and product acceptance criteria as guidelines. The output from A0 includes a high-level measurement process plan, a reaction plan, and support data. A high-level measurement process plan has different layers of information. The core defines measurement requirements. The first layer outside the core is a list of selected dimensional measurement equipment used to perform A0. The second layer is a sequence of high-level measurement operations. The third layer is a reaction plan and support data. A0 can be decomposed into five subactivities.

A1 Define Measurement Scope

This activity is to define scope for High-level Measurement Process Planning. A1 is further decomposed into three subactivities A11, A12 and A13.

A11 Identify Measurement Requirements

A11 is to identify the measurement requirements defined by the stakeholder's quality plan.  This typically identifies the tolerances and/or parameters while incorporating the accuracy requirements, production schedules, and sampling strategies required..

A12 Define Accuracy Requirements

Based on the product acceptance criteria, specify magnitudes of pass error and fail error on evaluating measurement features against their tolerance or dimension specifications.

A13 Define Measurement Task

A13 decomposes the measurement requirements into specific measurands to verify tolerances/parameters or establish datum features.  It also determines the measurement tasks that are to be performed to evaluate each measurement requirement. This activity requires sufficient product data, which should include geometric and topological information of a geometric model with associated tolerances, dimensions, and datum references.

A2 Determine Dimensional Measurement Equipment

It is an activity to determine all the measurement methods necessary measuring methods to perform measurement scope. Since it is on Layer One, A2 is optional. It can be decomposed into three subactivities.

A21 Select Coordinate Metrology Method

Select types/classes of coordinate metrology to perform the measurement.

A22 Select Sensors

Select types/classes of sensors that will be used with the selected coordinate metrology  types/classes to perform the measurement .

A23 Select Gages Method

Select types/classes of gages method to perform the measurement.

A3 Develop High-level Measurement Operation Plan

A3 is to develop a sequence of measurement operations. An operation is to perform an measurement task on a specific measurement  feature in a specific setup with identified dimensional measurement  equipment and predefined sample plan. A3 can be decomposed into four subactivities.

A31 Determine Setup and Fixturing

A31 is to determine part setups on a measurement  machine with proper fixturing to enable measurement  operation to be performed.

A32 Identified Features to be Measured

Based on the measurement requirements, A32 is to identify surface areas to be measured.  An identified surface area to be measured can also be referred to as an measurement feature.  A measured surface area is used to verify a tolerance or a dimension or to establish a datum.

A33 Develop Sampling Plan and Select Evaluation Functions

Develop a sampling plan for measuring all the identified measurement features and/or for selecting frequency of part to be measured in a batch, and select mathematical functions to evaluate sampled points.

A34 Determine High-Level Measurement Operations

A34 is to determine a sequence of operations based on measurement features, setups, and selected DME. Also, sampling strategy for a measurement feature will be described, as specified in the sampling plan developed in A33.

A4 Develop Reaction Plan

A4 is to develop a plan to react on possible measurement results.

A5 Generate Support Data

A5 is to generate necessary support data for a high-level measurement plan. Support data include, for example, due date, labor skill requirements, work order, and equipment order.

Input, output, mechanism/resource, and control data

Accuracy Requirements:

Requirement on magnitudes of pass error and fail error on all the inspected parts.

Batch size:

The quantity of parts in a batch of a job.

Coordinate Metrology Specification

Specific and/or classes of coordinate metrology that are needed to perform measurement, such as automatic touch probe CMM, manual scanning system, and (manual) laser tracker.

Criticality weighting:

Specific levels of criticality associated with each element within a set of parts or a set of features.

Dimensional Measurement Resource Database:

A database that contains information on measurement resources, such as coordination measuring machines, fixtures, rotary tables, sensors, and gages.

Fixture Knowledge Base:

Data on available fixturing resources used to assist an measurement  planner to generate setups and specify part fixturing methods with clamping devices.

Gage Specification:

Specific and/or classes of gages that are needed to perform measurement.

GD&T Standards:

National and international standards on dimensioning and geometric and statistical tolerancing.

High-Level Measurement Operation Plan:

A document that specifies a sequence of measurement features to be inspected, the setup sequence, a list of selected DME, and measurement  requirements.

High-level Measurement Process Plan:

A High-Level Measurement Process Plan is a specification of the measurement scope, which consists of parts and features with associated geometry, dimensions, and tolerances required for measurement.  A High-Level Measurement Process Plan will optionally specify the measurement method (measurement type + sensor type), a high-level measurement operations plan, a reaction plan, and support data.

Manufacturing Process Plan:

A set of instructions on sequence(s) of operations, tool selections, setup, and process parameters selection for a product manufacturing.

Measurement Criteria:

Company’s rules and regulations on measurement for measurement verification and/or product acceptance.

Measurement Features:

A surface area or a set of connected surface areas that will be in contact with a gage or sensed by a sensor in a measurement operation.

Measurement Knowledge Base:

A database that contains knowledge for determining how parts should be inspected.

Measurement Method Specification:::

A coordinate metrology specification / sensor specification pair.

Measurement Requirements:

Required instructions for inspecting manufactured parts. Required instructions include tolerances and dimensions to be verified and acceptable pass/fail errors.

Measurement Scope:

The specific subset of all the tolerances and dimensions on a manufactured part that need to be verified by a measurement.

Measurement Tasks

This is the specific measure task that must be performed to accommodate the measurement requirements.  The measurement task corresponds with a measurand, which associates a dme_measure_feature with a one or more tolerances or one or more datum reference frames.  A measurement task can be a Generate Measure Task, Evaluate Tolerance Measure Task, or Establish Datum Feature Measure Task.  Typical tasks might be to measure a 0.12 _hole_feature for general criteria (all corresponding tolerances and optional datum features), to measure the 3.5_hole_feature to evaluate its size_+/-0.20_tolerance, or to measure the bottom_planar_feature to establish the primary datum labels A for datum reference frame DRF_ABmC.

Part Characteristics:

Special attributes of a part that affect measurement process planning, such as sensor selection. Part characteristics include the material type of the part and surface characteristics. Surface characteristics include the reflectiveness and roughness of the surface of that is to be measured.

Product Definition:

All the necessary information on a product design for manufacturing and measurement planning. The information includes geometry, topology, tolerances, dimensions, and datum references.

Production Schedule:

Schedule of the production of a product, including measurement schedules.

Reaction Plan:

A document of reaction to possible measurement  results.

Quality Plan

A plan defining the quality requirements for a specific product.

Sampling Plan:

A document that specifies point locations and densities for measuring all the measurement features. On the part level, a sampling plan describes the frequency of measuring manufactured parts or the quantity of parts to be measured in a batch.

Sampling Statement:

A qualitative statement on sampling rate and sampling size.

Sensor Specification:

Specific and/or classes of sensors that are needed to perform measurement, such as touch probe sensor, touch scan sensor, white light scanner, and laser scan sensor.

Setups and Fixturing

A specification of setups needed to inspect all the tolerances and dimensions within the measurement scope and all the clamping devices that are needed in all the setups.

Standards for Quality:

International standards on quality assurance, such as standards in the ISO 9000 series.

Support Data:

Measurement  supporting information, such as job starting date, due date, work order, labor skill requirements, and equipment orders.